# Assignment 1 CIS 480/580

# Due Date: Monday, September 29th, 2025 Total points = 20

# Chapter 1:

Q1 (Q13 of textbook):

Name three appliances that are candidates for being run by an embedded CPU. (2 points)

Q2 (Q14 of textbook)

At a certain point in time, a transistor on a chip was 0.1 micron in diameter. According to Moore’s law, how big would a transistor be on next year’s model? (2 points)

# Chapter 2

Q3 (Q3 of textbook)

On a computer 1, all instructions take 10 nsec to execute. On computer 2, they all take 5 nsec to execute. Can you say for certain that computer 2 is faster? Discuss. Hint: pipeline. (2 points)

Q 4 (Q11 of textbook)

A certain computer can be equipped with 1,073,741,824 bytes of memory. Why would a manufacturer choose such a peculiar number, instead of an easy-to-remember number like 1,000,000,000? (2 points)

Q5 (not from textbook)

Device a (11,7) Hamming codeword for a 7-bit data field 1010101. Show and explain how the transmitted codeword will be generated? How will it be confirmed at the receiving end that no errors have occurred during the transmission? Show with an example, how errors and the locations of the errors will be detected and corrected. (6 points)

Q6 (Q14 of textbook)

In a Hamming code, some bits are “wasted” in the sense that they are used for checking and not for information. What is the percentage of wasted bits for messages whose total length (data + check bits) is 2n – 1? Evaluate this expression for values of n from 3 to 10. (3 points)

Q7 (Q17 of textbook) The disk, with 4096 bits/sector illustrated in Fig 2-19 of textbook/PowerPoint slides, has 1024 sector/track and a rotation rate of 7200 RPM. What is the sustained transfer rate of the disk over one track? (3 points)